Power grid reach vs settlement coverage map

Access to electricity is a foundational condition for economic activity, yet a significant proportion of settlements across Ghana remain beyond the practical service reach of the national high-voltage transmission network. You are required to perform a spatial analysis to identify those settlements by comparing the geographic extent of the existing transmission grid against the locations of all known populated settlements across the country. Settlements falling outside a 10-kilometre transmission line proximity threshold are classified as off-grid candidates, producing a spatially accurate, decision-ready dataset that energy planners, utility operators, and development partners can use to prioritise grid extension investments and off-grid electrification interventions where they are needed most.

You will be working with two layers already loaded in the project: elec_transmission.gpkg (a line layer of national high-voltage transmission corridors with voltage in kilovolts and operational status attributes), and settlements.gpkg (a point layer of 25,342 populated settlements carrying socioeconomic, infrastructure, and demand attributes). All the layers must be reprojected to EPSG:32630 (WGS 84 / UTM Zone 30N) before any processing begins, as all distance thresholds in this task are metric and Ghana falls within this UTM zone. The reprojected outputs are saved as elec_transmission_utm.gpkg, and settlements_utm.gpkg.

The core of this task is determining which settlements fall outside the effective grid corridor. Apply a 10,000-metre buffer to the entire reprojected transmission network and dissolve the result into a single unified polygon representing the on-grid service corridor. Save this as elec_transmission_10km_buffer.gpkg. The 10-kilometre threshold reflects a standard high-voltage grid proximity standard for energy access analysis and must not be altered.

Using that dissolved buffer polygon as the spatial reference, evaluate every settlement in settlements_utm.gpkg for whether its geometry falls within or outside the corridor. The output settlements_grid_tagged.gpkg must retain all 25,342 features from the input without exception and include a new text field named grid_access, populated with On-Grid for settlements inside the corridor and Off-Grid for those outside. Feature count parity between input and output is a hard requirement. From this tagged layer, produce settlements_off_grid.gpkg as an attribute-filtered subset containing only features where grid_access = 'Off-Grid'.

The settlements_grid_tagged.gpkg must be symbolised using grid_access as the classification field, green circles for on-grid settlements and red circles for off-grid settlements. Transmission lines are styled in orange, and the buffer polygon as a semi-transparent yellow fill. Save the completed project as Power-Grid-Reach-Coverage.qgz using relative paths to the same output folder (Power-Grid-Reach-Coverage, where all the mentioned input and derived layers are contained), ensuring no broken layer links on reopening.